## 1280 Truck In/Out

### Software User Guide

# Scope

The 1280 Truck In/Out application is included with the 1280 standard firmware and is loaded by importing the application from the Configuration menu “Import Files”.

Sequence of Operation

The touch buttons on power up and exit configuration are Inbound Trucks, Toggle Lights, and Setup.  System streams the current weight to a stop n go remote display.  The Toggle Lights only change the lights if Traffic Lights is set to Manual in the Setup menu.  If Traffic Lights are set to Auto they change based on the sequence below and the Toggle Lights widgets are for visual only.

- With the traffic light Green the truck will pull fully onto the scale for a transaction.

- The traffic light will turn Red when the Threshold Weight is exceeded.

- System displays Inbound Trucks and Toggle Light Buttons.

Weigh In

- System showing Green on the traffic signal.

- Driver pulls the truck fully onto the scale and stops his truck next to a kiosk.

- System shows Weigh In and Weigh Out buttons once the scale weight exceeds the Threshold Weight. Traffic signals will change to Red.

- Operator presses the Weigh In button.

- System prompts – “Enter Truck ID”.

- Operator enters the TRUCK ID and presses the ENTER key.

- System queries the Inbound database table.

  - ID FOUND:

    - System displays – “Truck Already Weighed In, Update Tare” and Yes No touch widgets

      - If No is pressed system goes back to step 3.

      - If Yes is pressed system goes to step7.b but update inbound database.

  - ID NOT FOUND:

    - Captures a stable GROSS weight from the Total Scale.

    - Adds inbound transaction to the truck register.

    - Prints a weigh in ticket. A Reprint button is available until the truck exits the scale.

    - If Truck Exit Required:

      - System turns the light Green.

      - Displays – “Weigh In Complete – Exit Scale”.

    - If Truck Exit Disabled:

      - Displays – “Weigh In Complete”

      - Goes back to STEP 3.

- When the weight drops below the Threshold Weight system returns to STEP 1.

Weigh Out

- System showing Green on the traffic signal.

- Driver pulls the truck fully onto the scale and stops his truck next to a kiosk.

- System shows Weigh In and Weigh Out buttons once the scale weight exceeds the Threshold Weight. Traffic signals will change to Red.

- Operator presses the Weigh Out button.

- Operator presses the Weigh Out button.

- System prompts – “Enter Truck ID”.

- Operator enters a TRUCK ID and presses the ENTER key.

- System queries the Inbound database table.

  - ID NOT FOUND with NO KEYED Tare

    - Momentary displays – “Truck NOT Found” and goes back to STEP 3.

  - ID NOT FOUND with KEYED tare

    - System prompts – “Enter Product ID” only if there are products stored in the database.  If only one product exists system will automatically go to customer prompt.

    - Operator does one of the following:

      - Operator enters a stored PRODUCT ID and presses the ENTER key.

      - Operator press ENTER to skip;

        - System displays a list of all Products in the Product database table as well as the Page Up - Page Down touch widgets.

        - Operator utilizes the Page Up - Page Down touch widgets to scroll through the database and select the product to be processed. If Exit is pressed the system takes no action and returns to the Enter Product Id prompt.

    - System prompts – “Enter Customer ID” only if there are customers stored in the database.  If only one customer exists system will automatically use that customer and go to the next STEP.

    - Operator does one of the following:

      - Operator enters a stored CUSTOMER ID and presses the ENTER key.

      - Operator press ENTER to skip;

        - System displays a list of all Customers in the Customer database table as well as the Page Up - Page Down touch widgets.

        - Operator utilizes the Page Up - Page Down touch widgets to scroll through the database and select the customer to be processed. If Exit is pressed the system takes no action and returns to the Enter Customer Id prompt.

    - Captures a stable NET weight from the Total Scale.

    - Prints a Weigh Out Ticket.

    - Stores Transaction Record.

    - Increments a sequential Ticket Number. Resets at 10,000.

    - System turns the light Green.

    - If Truck Exit Required:

      - Displays – “Weigh Out Complete – Exit Scale”.

    - If Truck Exit Disabled:

      - Displays – “Weigh Out Complete” and goes back to step 3.

  - ID FOUND without KEYED tare

    - System prompts – “Enter Product ID” only if there are products stored in the database.  If only one product exists system will automatically go to customer prompt.

    - Operator does one of the following:

      - Operator enters a stored PRODUCT ID and presses the ENTER key.

      - Operator press ENTER to skip;

        - System displays a list of all Products in the Product database table as well as the Page Up - Page Down touch widgets.

        - Operator utilizes the Page Up - Page Down touch widgets to scroll through the database and select the product to be processed. If Exit is pressed the system takes no action and returns to the Enter Product Id prompt.

    - System prompts – “Enter Customer ID” only if there are customers stored in the database.  If only one customer exists system will automatically use that customer and go to the next STEP.

    - Operator does one of the following:

      - Operator enters a stored CUSTOMER ID and presses the ENTER key.

      - Operator press ENTER to skip;

        - System displays a list of all Customers in the Customer database table as well as the Page Up - Page Down touch widgets.

        - Operator utilizes the Page Up - Page Down touch widgets to scroll through the database and select the customer to be processed. If Exit is pressed the system takes no action and returns to the Enter Customer Id prompt.

    - Captures a stable GROSS weight from the Total Scale.

    - Does Value Swapping if enabled in the Setup Menu (Not with KEYED Tare).

    - Prints a Weigh Out Ticket. A Reprint button is available until the truck exits the scale.

    - System adds a new record to the Transaction database table. (Deletes 25% of oldest records when full)

    - Increments a sequential Ticket Number. Resets at 10,000.

    - System turns the light Green.

    - Deletes inbound weight if Stored Tares is disabled in the Setup Menu (Not with KEYED Tare).

    - Deletes inbound weight if Stored Tares is enabled and Truck ID has a decimal in it.

    - If Truck Exit Required:

      - Displays – “Weigh Out Complete – Exit Scale”.

    - If Truck Exit Disabled:

      - Displays – “Weigh Out Complete” and goes back to step 3.

- When the weight drops below the Threshold Weight system returns to STEP 1.

Inbound Trucks

- Operator presses the Inbound Trucks button.   If a password is stored system prompt enter password before display inbound weighments.

- The display will show 8 inbound weighments, including ID, weight and time/date and the buttons will switch to Page Up, Page Down, Delete, Delete ALL, and Exit.

- Press the Page Up and Page Down button to scroll through stored inbound weighments 8 at a time.

- Operator touches inbound weighment to delete then presses the Delete button to delete the selected inbound weighment.  The display will display “Are You Sure?” and change the buttons to Yes and No.  To delete press the Yes button.

- Press the Delete All button to delete all inbound weighments.  The display will prompt “Are You Sure?” and change the button to Yes and No.  To clear all weighments press the Yes button.

# Serial Communications – Tickets, PC

TRUCK ID 555

GROSS  6000 lb INBOUND

08:43 AM 11/30/2016

Weigh In Ticket (Auxfmt1)

The program uses the Auxiliary Print Format #1 so that the dealer may modify the print format thru the 1280 front panel or by using Revolution PC software.

TICKET 1

TRUCK ID 555
PRODUCT  444 APPLES (only prints entire line if entered)
CUSTOMER 333 JOHN DOE (only prints entire line if entered)

GROSS  100000 lb
TARE     6000 lb RECALLED
NET     94000 lb

47.0 tn (only prints if product selected)

09:43 AM 11/30/2016

Weigh Out Ticket (Auxfmt2)

The program uses the Auxiliary Print Format #2 so that the dealer may modify the print format thru the 1280 front panel or by using Revolution PC software.

Customer by Product Report

Printed @ 12:17 PM 12/02/2016

1 XYZ Trucking

Tin              40000 lb

Gold             20000 lb

Bismuth          20000 lb

2 ABC Trucking

Tin              20000 lb

Gold             60000 lb

3 Rice Lake Weigh

Tin              20000 lb

Platinum         40000 lb

Bismuth          20000 lb

4 Intruder

Tin              20000 lb

Platinum         20000 lb

Lead             40000 lb

5 Mattson

Platinum         20000 lb

Bismuth          40000 lb

Customer by Product Report

The report is not editable without an iRite program modification.

Customer by Truck Report

The report is not editable without an iRite program modification.

Customer by Truck Report

Printed @ 12:18 PM 12/02/2016

1 XYZ Trucking

F1               40000 lb

S1               40000 lb

2 ABC Trucking

F1               40000 lb

S1               40000 lb

3 Rice Lake Weigh

C1               20000 lb

F1               20000 lb

S1               40000 lb

4 Intruder

C1               20000 lb

F1               20000 lb

S1               40000 lb

5 Mattson

F1               20000 lb

S1               40000 lb

Rice Lake Stop n Go Green Light (AuxFmt19)

The message will be transmitted (Defaulted to Port 2).  The format can be modified through the 1280 front panel or using a PC application called Revolution.

Rice Lake Stop n Go Red Light (AuxFmt20)

The message will be transmitted (Defaulted to Port 2).  The format can be modified through the 1280 front panel or using a PC application called Revolution.

Application Setup & Configuration

The Setup button is password-protected and offers access to the following:

- Display Program Name & Version and Display a weight widget

| Parameter | Default | Button | Description |
| --- | --- | --- | --- |
| System Password | “” | Setup Password | Changing the password that is required for entry into the Setup menu. Setting the password to nothing will cause the system to not prompt for a password when the [Setup Menu] touch widget is pressed. |
| Inbound Trucks Password | “” | Trucks Password | Changing the password that is required for entry into the Inbound Trucks display. Setting the password to nothing will cause the system to not prompt for a password when the [Inbound Trucks] touch widget is pressed. |
| Ticket Number | 1 | Ticket Number | Sequential number assigned to each weigh out transaction. |
| Stored Tares | ON / OFF | Stored Tares | When this is ON the Inbound Weight is stored even after Weighing Out. When this is OFF the Inbound record is deleted after Weighing Out. |
| Value Swapping | ON / OFF | Value Swapping | When this is ON the Gross weight is always the larger of the Gross and Tare values, whether the Outbound weight was larger or smaller than the Inbound weight. When this is OFF the Gross is always the Outbound weight. |
| Threshold Weight | 5000 lb | Threshold Weight | Enter the weight that triggers the optional traffic light, and also is what must go above/below between weighments are allowed. |
| Import/Export Data |  | Import/Export | 1280 standard function. Please reference the 1280 Installation manual for more details. Allows the operator to import Products into the Product database table from a USB flash drive using .DB file type and allows the user to save transactions to the USB flash drive. |
| Add/Edit Products | - | Add//Edit Product | Allows the operator to Add/Edit products to the Product database table. |
| Delete Products | - | Delete Product | Allows the operator to Delete products in the Product database table. |
| Add/Edit Customers | - | Add//Edit Customer | Allows the operator to Add/Edit customers to the Customer database table. |
| Delete Customers | - | Delete Customer | Allows the operator to Delete customers in the Customer database table. |
| Print Reports | - | Customer by Product Customer by Truck | Allows operator to print two totals reports (totals only). |
| Truck Exit | Required / Disabled | Truck Exit | Allow operator to decide if truck has to leave scale in between weighments. |
| Traffic Lights | Auto / Manual | Traffic Lights | Allows operator to decide if the traffic lights are controlled by the program sequence (Auto) or by the touch widgets (Manual). |
| iQube2 Diagnostics | - | iQube2 Diagnostics | Displays 1280 built in iQube2 Diagnostics touch widget. An iQube2 cannot be configured to Port 1. If configured to Port 2 make sure streaming is OFF, Auxiliary Formats #19 and #20 are set to Port 0. |
| Clear Transaction |  | Clear Transaction | Allow the operator to clear transaction database with a Yes or No Prompt |
| Inbound Tickets | 1 | Inbound Ticket | Allow operator to change how many inbound tickets are printed |
| Outbound Tickets | 1 | Outbound Tickets | Allow operator to change how many outbound tickets are printed |

Database Tables

Report Database is used for Reporting in the iRite program.

| Inbound Database Table – 1,000 Records |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ID | String | Alphanumeric ID – 15 Characters |
| InPWgt | Real | Primary Inbound weight stored |
| InSWgt | Real | Secondary Inbound weight stored |
| InTWgt | Real | Tertiary Inbound weight stored |
| DT | Datetime | Time and Date of transaction |

| Product Database Table – 100 Records |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ID | String | Alphanumeric ID – 15 Characters |
| Name | String | Product Name – 15 Characters |
| CUnits | String | What to print for converted units |
| CMult | Real | Convert to second units (tn or bushels) |

| Customer Database Table – 100 Records |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ID | String | Alphanumeric ID – 15 Characters |
| Name | String | Customer Name – 15 Characters |

| Transaction Database Table – 5000 Records |  |  |
| --- | --- | --- |
| Field | Type | Description |
| Ticket | Integer | Sequential Ticket Number |
| TruckID | String | Truck ID – 15 Characters |
| ProdID | String | Product ID – 15 Characters |
| ProdName | String | Product Name tied to ID |
| CustID | String | Customer ID – 15 Characters |
| CustName | String | Customer Name tied to ID |
| Gross | Real | Gross Weight |
| Tare | Real | Tare Weight |
| Net | Real | Net Weight |
| ConNet | Real | Converted units based on product conversion value |
| OutUnits | String | Outbound Unit String |
| InDT | Datetime | Date and Time of Inbound |
| OutDT | DateTime | Date and Time of Outbound |

Hardware Setup

| Option Card Locations |  |
| --- | --- |
| Slot | Type |  |
| 1 | Single Channel A/D Card |  |
| 2 | Dual Channel A/D Card (Optional) |  |

| Digital I/O |  |  |  |
| --- | --- | --- | --- |
| Slot | Bit | Type | Function |
| 0 | 1 | Output | Green Traffic Light |
| 0 | 2 | Output | Red Traffic Light |
| 0 | 3-8 | Off | Currently Not used |

| Serial Port |  |  |  |
| --- | --- | --- | --- |
| Port | Type | Description | Setup |
| 1 | CMD | Ticket Printer | 9600,8,N,1 |
| 2 | CMD | *Rice Lake Stop and Go Remote Default for Legal For Trade Streaming | 9600,8,N,1 |

*To configure another device on Port 2 please do the following:

- In Stream Formatting deselect Stream Destination Serial Port 2.

- Ticket Formatting set Destination Port to None.

- Ticket Printer on Port 1 cannot be removed.

# Version Changes

| Version | Changes | Date | Firmware |
| --- | --- | --- | --- |
| 1.00 | - Initial Release | 3/1/2015 | 1.00 |
| 1.01 | - Added 2 A/Ds for a 3-scale + Total system - Added Product and Customer prompts, databases, and ticket output. - Added Two Totals Reports - Added Setup Menu and Weigh/Out touch buttons. - Added Import/Export option. - Added Transaction database with auto delete when full. | 12/1/2016 | 1.05 |
| 1.02 | - Removed Softkey - Inbound Database to 1000 trucks - Add ability to weigh in/out without leaving scale | 04/7/2017 | 1.06 |
| 1.03 | - Linked internal images/buttons though Revolution file. - Added support for 12 inch display (4 – Revolution files) | 10/17/2017 | 1.07 |
| 1.04 | - Addressed a Exit widget issue | 11/28/2017 | 1.07 |
| 1.05 | - Added iQube2 Diagnostics button to all 4 Revolution file configurations. | 1/31/2018 | 1.07.01 |
| 1.06 | - Added ability to store Primary, Secondary and Tertiary weights in the Inbound Database while processing an inbound truck. This allows an operator to weigh out in different units than inbound. - Added a Traffic Light option. The Toggle Lights only change the lights if Traffic Lights is set to Manual. If Traffic Lights is set to Auto they change based on the regular sequence and the Toggle Lights widgets are visual only. - Unlocked PRINT and UNITS key to allow the operator to print a gross ticket or toggle between units all the time. - Added units string to Transaction database. | 09/07/2018 | 1.09 |
| 1.07 | - Addressed a prompting issue when entering more than 15 characters on the Truck ID Prompt. - Changed 7 inch iQube2 port 2 baud to 115200. | 04/03/2018 | 1.09 |
| 1.08 | - Added Display List options for Customer and Product selection - Addred Reprint Option - Added a Sequential Ticket Number | 07/10/2020 | 1.11 |
| 1.09 | - Add ability to update tare on a weigh in. - Update Rev files to latest modules - Fix to Keyed tare option - Fix to Page up and down - Add Inbound and Outbound number of tickets - Add Clear Transaction to setup menu | 08/24/2023 | 2.05 |
| Download: SC N TF SP W DB COD SC = Standard Configuration N = Network Configuration TF = Ticket Formats SP = Setpoints W = Widgets DB = Database Tables COD = Program |  |  |  |